# 26 ft box truck pre-trip inspection checklist

General information, not legal advice. Rule citations as checked October 2026. Source: boxtruckdispatch.com/guides/box-truck-inspection-checklist

Date: ______________________________

Truck / unit number: ______________________________

Odometer: ______________________________

Driver: ______________________________

| Paperwork in the cab |  |
| --- | --- |
| OK [ ] Defect [ ] | Registration and insurance card for this truck |
| OK [ ] Defect [ ] | Proof of annual inspection (report or decal) on the truck (49 CFR 396.17(c)) |
| OK [ ] Defect [ ] | Last DVIR reviewed and signed if it listed defects (49 CFR 396.13) |
| OK [ ] Defect [ ] | Medical card (interstate CMV drivers) (49 CFR 391.41) |
| OK [ ] Defect [ ] | ELD data transfer and malfunction instruction sheets, 8 days of blank log sheets (49 CFR 395.22(h)) |
| OK [ ] Defect [ ] | Rate con, BOL and pickup number for today's load |
| Engine compartment |  |
| OK [ ] Defect [ ] | Oil, coolant, power steering and washer fluid levels |
| OK [ ] Defect [ ] | Belts tight, not cracked or frayed |
| OK [ ] Defect [ ] | Hoses not cracked, bulging or leaking |
| OK [ ] Defect [ ] | No fresh leaks under the truck (oil, coolant, fuel, brake fluid) |
| In the cab |  |
| OK [ ] Defect [ ] | Gauges and warning lights normal after start-up |
| OK [ ] Defect [ ] | Horn works (49 CFR 392.7) |
| OK [ ] Defect [ ] | Wipers and washers work; blades not torn (49 CFR 392.7) |
| OK [ ] Defect [ ] | Windshield free of cracks in the wiper sweep area (49 CFR 396 app. A) |
| OK [ ] Defect [ ] | Mirrors clean, adjusted, not cracked (49 CFR 392.7) |
| OK [ ] Defect [ ] | Steering: no excess play, no binding (49 CFR 392.7) |
| OK [ ] Defect [ ] | Parking brake holds the truck in gear against light throttle (49 CFR 392.7) |
| OK [ ] Defect [ ] | Service brakes stop the truck straight at low speed (49 CFR 392.7) |
| OK [ ] Defect [ ] | Seat belt latches and is not frayed (49 CFR 392.16) |
| OK [ ] Defect [ ] | Heater and defroster work |
| Lights and reflectors |  |
| OK [ ] Defect [ ] | Headlights, high and low beam (49 CFR 392.7) |
| OK [ ] Defect [ ] | Turn signals and four-way flashers, front and rear (49 CFR 392.7) |
| OK [ ] Defect [ ] | Brake lights and tail lights (49 CFR 392.7) |
| OK [ ] Defect [ ] | Clearance and identification lights on the box (49 CFR 392.7) |
| OK [ ] Defect [ ] | Reflectors and reflective tape clean and in place (49 CFR 392.7) |
| Tires, wheels and suspension |  |
| OK [ ] Defect [ ] | Steer tires: at least 4/32 in tread, no cuts or bulges (49 CFR 393.75) |
| OK [ ] Defect [ ] | Drive tires: at least 2/32 in tread, duals not touching (49 CFR 393.75) |
| OK [ ] Defect [ ] | Inflation checked with a gauge, valve caps on (49 CFR 392.7) |
| OK [ ] Defect [ ] | Lug nuts tight; no rust streaks or missing nuts (49 CFR 392.7) |
| OK [ ] Defect [ ] | Hub seals not leaking |
| OK [ ] Defect [ ] | Springs, shocks and air bags not broken or leaking (49 CFR 396 app. A) |
| Cargo box and door |  |
| OK [ ] Defect [ ] | Roll-up door opens, closes and latches; pull strap in place |
| OK [ ] Defect [ ] | Door seals and roof intact, no light showing through (water claims) |
| OK [ ] Defect [ ] | Floor solid, no holes or soft spots |
| OK [ ] Defect [ ] | E-track, load bars and straps on board and in good shape (49 CFR 393.100) |
| OK [ ] Defect [ ] | Rear impact guard (bumper) not bent or loose (49 CFR 396 app. A) |
| OK [ ] Defect [ ] | Cargo light works |
| Liftgate (if equipped) |  |
| OK [ ] Defect [ ] | Runs a full cycle: down, up, fold or stow |
| OK [ ] Defect [ ] | No hydraulic leaks at the pump, hoses or cylinders |
| OK [ ] Defect [ ] | Chains, cables and pins secure; platform not cracked |
| OK [ ] Defect [ ] | Capacity decal readable; never load past it |
| OK [ ] Defect [ ] | Controls and safety latches work; gate stowed and latched before driving |
| Reefer unit (if equipped) |  |
| OK [ ] Defect [ ] | Unit fuel and oil level |
| OK [ ] Defect [ ] | Pre-cooled to the setpoint on the rate con before loading |
| OK [ ] Defect [ ] | Temperature recorder running |
| Emergency equipment |  |
| OK [ ] Defect [ ] | Fire extinguisher rated 5 B:C or more (or two 4 B:C), charged and securely mounted (49 CFR 393.95(a)) |
| OK [ ] Defect [ ] | Three bidirectional reflective warning triangles (49 CFR 393.95(f)) |
| After loading and at day's end |  |
| OK [ ] Defect [ ] | Freight blocked, braced or strapped so it cannot shift (49 CFR 393.100) |
| OK [ ] Defect [ ] | Weight on the BOL within your payload and GVWR |
| OK [ ] Defect [ ] | Door locked or sealed; seal number matches the BOL |
| OK [ ] Defect [ ] | End of day: write up every defect on a DVIR and sign it (49 CFR 396.11) |

Defects found and action taken:

____________________________________________________________________________

____________________________________________________________________________

Driver signature: ______________________________

Repairs certified by (if defects listed): ______________________________